# Small-UAS Industrial Base

# Supply Chain Gap Analysis

Pursuant to Section 914, FY2026 National Defense Authorization Act

Prepared by: Rate Manufacturing LLC

Decatur, Alabama

December 2025

Version 1.0 - DRAFT FOR WORKING GROUP REVIEW

# Executive Summary

This analysis examines the current capacity of the United States domestic industrial base to support the DoD Drone Dominance Program requirement of 340,000+ small unmanned aircraft systems (sUAS). The analysis is structured around the nine critical component categories defined in Section 914 of the FY2026 NDAA.

## Key Findings

- Critical Gap: US domestic capacity can currently support approximately 5-10% of projected demand across most component categories

- Brushless Motors: Only 2-3 domestic manufacturers exist; combined capacity ~50,000-100,000 units/year vs. 1,360,000 required

- Flight Controllers: Blue UAS Framework lists only 3-4 domestic assemblers; raw component sourcing remains Asia-dependent

- Batteries: Zero domestic cell manufacturing; 100% dependent on allied nations (Taiwan, South Korea)

- Rare Earth Magnets: Active DOD/DOE investment; USA Rare Earth (OK), Noveon (TX) coming online 2026

Bottom Line: Without significant infrastructure investment and capacity expansion, the US cannot achieve supply chain independence for the Drone Dominance Program within the projected timeline.

# Section 914: Small-UAS Industrial Base Working Group

Section 914 of the FY2026 NDAA establishes the Small-UAS Industrial Base Working Group, directed to analyze the supplier base and recommend investments to remediate fragile supply chains. This gap analysis directly supports the Working Group's mandate.

## Statutory Component Categories

Section 914(g)(2) defines 'sUAS component' as the following critical items:

- (A) Brushless motors

- (B) Batteries

- (C) Antennae

- (D) Flight controllers, including printed circuit boards

- (E) Wiring harnesses

- (F) Rotors

- (G) Blades and propellers

- (H) Chassis, bodies, and frames

- (I) Sensors, including electro-optical and infra-red sensors, GPS, and other such sensors

## Working Group Composition

Per Section 914(b), the Working Group includes:

- Deputy Secretary of Defense (Chair)

- Assistant Secretary of Defense for Industrial Base Policy

- Director, Defense Autonomous Warfare Group

- Defense Innovation Unit (DIU) representatives

- Service acquisition executives (Army, Navy, Air Force)

- Army Materiel Command representatives

- US Special Operations Command representatives

Initial Report Due: April 1, 2026

# Demand Analysis: 340,000 Drone Requirement

The Drone Dominance Program targets procurement of 340,000+ sUAS across FY2026-FY2030. This creates the following component demand:

| Component Category | Per Drone | Total Demand | Annual Rate |
| --- | --- | --- | --- |
| Brushless Motors | 4 | 1,360,000 | 340,000/yr |
| Batteries (cell level) | 16-24 cells | 5.4-8.2M cells | 1.4-2M/yr |
| Flight Controllers | 1 | 340,000 | 85,000/yr |
| ESCs (4-in-1) | 1 | 340,000 | 85,000/yr |
| GPS/GNSS Modules | 1 | 340,000 | 85,000/yr |
| Carbon Fiber Frames | 1 | 340,000 | 85,000/yr |
| Propellers (sets) | 2 sets | 680,000 sets | 170,000/yr |
| Cameras/Sensors | 1-2 | 340,000-680,000 | 85,000-170,000/yr |
| Antennas (RF, GPS) | 3-4 | 1,020,000-1,360,000 | 255,000-340,000/yr |

# Component-by-Component Gap Analysis

## (A) Brushless Motors — CRITICAL GAP

Status: 92-96% CAPACITY SHORTFALL

Brushless motors are the most critical supply chain gap. The 340,000 drone program requires 1,360,000 motors over 4 years. Current US domestic capacity is approximately 50,000-100,000 units annually.

### Verified US Motor Manufacturers

| Company | Location | Est. Capacity | Status |
| --- | --- | --- | --- |
| UMAC/Rotor Riot | Orlando, FL | ~20,000/yr | Blue UAS Listed; scaling up |
| KDE Direct | Bend, OR | ~15,000/yr | USA designed/assembled; sources components |
| Vertiq | USA | ~5,000/yr | Blue UAS Listed; integrated modules |
| Rate Manufacturing | Decatur, AL | Building | Vertical integration; domestic magnets |

### Motor Sub-Component Suppliers (USA)

| Component | USA Suppliers | Location | Notes |
| --- | --- | --- | --- |
| N52H Magnets | USA Rare Earth | Stillwater, OK | Q1 2026 production |
|  | Noveon Magnetics | San Marcos, TX | Currently producing DFARS |
|  | Arnold Magnetic Technologies | Rochester, NY | Full NdFeB range |
|  | Dura Magnetics | Sylvania, OH | Custom N52H |
| Laminations | Thomson Lamination (TLC) | Maple Shade, NJ | AS9100; 1500+ dies |
|  | Laser Technologies Inc | Naperville, IL | ITAR/NADCAP |
|  | Polaris Laser Laminations | West Chicago, IL | ITAR certified |
| Magnet Wire | Remington Industries | Johnsburg, IL | Class H/ML polyimide |
|  | MWS Wire Industries | Westlake Village, CA | Aerospace grades |
| Bearings | Boca Bearings | Boynton Beach, FL | Miniature 686ZZ ABEC-7 |
|  | Schatz Bearing | Poughkeepsie, NY | Aerospace precision |
| Ti Shafts | UPMET | Wallingford, CT | Ti-6Al-4V aerospace |
| CNC (Bells/Bases) | SendCutSend | Reno, NV | 7075/6061 aluminum |

## (B) Batteries — CRITICAL GAP

Status: 100% ALLIED NATION DEPENDENCY

There is zero domestic lithium-ion cell manufacturing capacity. All high-performance cells (Molicel P42A, Samsung 50E, LG M50) are manufactured in Taiwan, South Korea, or Japan. These are allied nations, making cells NDAA-compliant, but not supply-chain secure.

| Manufacturer | Location | NDAA Status | Notes |
| --- | --- | --- | --- |
| Molicel (P42A) | Taiwan | Allied | Preferred for FPV; 4200mAh 45A |
| Samsung SDI (50E) | South Korea | Allied | High energy density |
| LG Chem (M50) | South Korea | Allied | 5000mAh high capacity |
| CATL, EVE, etc. | China | PROHIBITED | Covered foreign entity |

Recommendation: Support domestic battery assembly from allied-nation cells. Monitor DOE investments in domestic cell manufacturing (Panasonic Nevada, SK Battery Georgia).

## (D) Flight Controllers — MODERATE GAP

Status: USA ASSEMBLY EXISTS; COMPONENT SOURCING GAP

| Company | Products | Status | Notes |
| --- | --- | --- | --- |
| ARK Electronics | FC, ESC, GPS, IMU | Blue UAS | Primary Blue UAS supplier |
| Rotor Riot/UMAC | Brave F7 FC, 55A ESC | Blue UAS | Ewing Aerospace USA mfg |
| Auterion | Skynode Mission Computer | Blue UAS | PX4-based systems |
| ModalAI | VOXL 2, Power Module | Blue UAS | AI/autonomy focus |

Gap: MLCCs (capacitors), quartz crystals, and semiconductor dies are still 100% Asia-sourced. 'USA assembly' does not equal supply chain security.

## (H) Chassis, Bodies, and Frames — ADEQUATE DOMESTIC CAPACITY

Status: DOMESTIC MANUFACTURING EXISTS

| Company | Location | Products | Notes |
| --- | --- | --- | --- |
| Armattan Productions | Tallahassee, FL | CF Frames | USA CNC carbon fiber; lifetime warranty |
| Catalyst Machineworks | USA | Custom CF | Custom fabrication available |
| DragonPlate | Elbridge, NY | CF Sheet/Plate | USA made carbon fiber materials |
| RockWest Composites | West Jordan, UT | CF Composites | Full composites capability |

Assessment: Frame manufacturing is the strongest domestic capability. Capacity expansion is feasible with standard industrial investment.

## (G) Blades and Propellers — MODERATE GAP

Status: LIMITED USA MANUFACTURING

| Company | Location | Products | Notes |
| --- | --- | --- | --- |
| APC Propellers | Woodland, CA | Injection molded | USA made; injection molded |
| KDE Direct | Bend, OR | CF Propellers | USA made carbon fiber |
| Gemfan (via distributors) | Taiwan | Tri-blade props | Allied nation; dominant market share |

# Capacity Gap Summary

| Component | Demand (4yr) | US Capacity/yr | Gap | Status |
| --- | --- | --- | --- | --- |
| Brushless Motors | 1,360,000 | ~75,000 | 95% | CRITICAL |
| Battery Cells | 5.4-8.2M | 0 | 100% | CRITICAL |
| Flight Controllers | 340,000 | ~50,000 | 85% | HIGH |
| ESCs | 340,000 | ~50,000 | 85% | HIGH |
| GPS Modules | 340,000 | ~30,000 | 91% | HIGH |
| Carbon Fiber Frames | 340,000 | ~25,000 | 70% | MODERATE |
| Propellers | 680,000 sets | ~50,000 | 93% | HIGH |
| Cameras/Sensors | 340,000+ | ~10,000 | 97% | CRITICAL |
| Antennas | 1M+ | ~100,000 | 90% | MODERATE |

# Recommendations for Working Group

## Immediate Actions (0-6 months)

- Establish demand signal clarity: Issue firm commitments for Phase I-IV quantities to enable supplier capital investment

- Accelerate magnet supply chain: Prioritize DOD/DOE support for USA Rare Earth (OK), Noveon (TX) to reach production volumes

- Qualify additional lamination suppliers: Thomson, Polaris, Laser Technologies have capacity; need drone-specific tooling

## Near-Term Actions (6-18 months)

- Establish motor manufacturing hubs: Consider SkyFoundry model for GOGO or GOCO motor production facilities

- Fund battery assembly capability: Domestic pack assembly from allied-nation cells reduces risk

- Expand Blue UAS Framework: Increase component-level (not just system-level) compliance verification

## Long-Term Actions (18+ months)

- MLCC reshoring: Coordinate with CHIPS Act programs for defense-priority MLCC capacity

- Battery cell manufacturing: Monitor DOE investments; consider defense-specific cell format development

- Sensor/camera domestic production: Investigate domestic CMOS sensor capability with defense funding

# Distribution List

This gap analysis is intended for distribution to the following stakeholders per Section 914:

## Section 914 Working Group Members

- Office of the Deputy Secretary of Defense

- Assistant Secretary of Defense for Industrial Base Policy (ASD(IBP))

- Director, Defense Autonomous Warfare Group

- Defense Innovation Unit (DIU)

- Army Service Acquisition Executive (ASAE)

- Navy Service Acquisition Executive

- Air Force Service Acquisition Executive

- Army Materiel Command (AMC)

- US Special Operations Command (USSOCOM)

## Congressional Defense Committees

- House Armed Services Committee (HASC)

- Senate Armed Services Committee (SASC)

- House Appropriations Committee, Defense Subcommittee

- Senate Appropriations Committee, Defense Subcommittee

## Other Relevant Stakeholders

- Office of Strategic Capital

- Army Futures Command

- PEO Aviation (PBAS Program Office)

- Department of Energy (Critical Minerals programs)

- Defense Logistics Agency

# Appendix A: Complete Supplier Database

A comprehensive database of 120+ NDAA/Blue UAS compliant suppliers has been compiled and is available in CSV format upon request. Contact: Rate Manufacturing LLC

## Data Sources

- DIU Blue UAS Framework Approved Products List

- Direct supplier verification and quotation

- DFARS/NDAA compliance certifications

- Bill of Materials analysis from prototype drone builds

- Industry interviews and trade publications

— END OF DOCUMENT —